DEPARTMENT OF LICENSING AND REGULATORY AFFAIRSLABOR AND ECONOMIC OPPORTUNITY

DIRECTOR’S OFFICE

CONSTRUCTION SAFETY AND HEALTH STANDARD

Filed with the secretary of state on

These rules take effect immediately upon filing with the secretary of state unless adopted under section 33, 44, or 45a(6) of the administrative procedures act of 1969, 1969 PA 306, MCL 24.233, 24.244, or 24.245a.  Rules adopted under these sections become effective 7 days after filing with the secretary of state.

(By authority conferred on the director of the department of licensing and regulatory affairslabor and economic opportunity by sections 14, 16, 19, 21, and 24 of the Michigan occupational safety and health act, 1974 PA 154, MCL 408.1014, 408.1016, 408.1019, 408.1021, and 408.1024, and Executive Reorganization Order Nos. 1996-1, 1996-2, 2003-1, 2008-4, and 2011-4, and 2019-3, MCL 330.3101, 445.2001, 445.2011, 445.2025, and 445.2030, and 125.1998)

R 325.60501 of the Michigan Administrative Code is amended, as follows:

CONSTRUCTION SAFETU AND HEALTH STANDARD

PART 605. METHYLENEDIANILINE (MDA) IN CONSTRUCTION

R 325.60501 Scope, application, adoption, and availability of standards.

Rule 501. (1) These rules apply to all construction operations as defined in the Michigan occupational safety and health act (MIOSHA), 1974 PA 154, MCL 408.1001 to 408.1094, in which there is exposure to methylenedianiline (MDA), including but not limited to, all of the following:

(a) Construction, alteration, repair, maintenance, or renovation of structures, substrates, or portions thereof, that contain MDA.

(b) Installation or the finishing of surfaces with products containing MDA.

(c) MDA spill or emergency cleanup, or both, at construction sites.

(d) Transportation, disposal, storage, or containment of MDA or products containing MDA on the site or location at which construction activities are performed.

(2) Except as provided in subrule (7) of this rule and 29 CFR 1926.60(f)(5), these rules do not apply to the processing, use, and handling of products containing MDA if initial monitoring indicates that the product is not capable of releasing MDA in excess of the action level under the expected conditions of processing, use, and handling that will cause the greatest possible release, and if no "dermal exposure to MDA" can occur.

(3) Except as provided in subrule (7) of this rule, these rules do not apply to the processing, use, and handling of products containing MDA if objective data are reasonably relied upon that demonstrate the product is not capable of releasing MDA under the expected conditions of processing, use, and handling that will cause the greatest possible release, and if no "dermal exposure to MDA" can occur.

(4) Except as provided in subrule (7) of this rule, these rules do not apply to the storage, transportation, distribution, or sale of MDA in intact containers sealed in such a manner as to contain the MDA dusts, vapors, or liquids, except for the provisions of Construction Safety Standard Part 42. “Hazard Communication,” R 408.44201 to R 408.44204, and 29 CFR 1926.60(e).

(5) Except as provided in subrule (7) of this rule, these rules do not apply to materials in any form that contain less than 0.1% MDA by weight or volume.

(6) Except as provided in subrule (7) of this rule, these rules do not apply to "finished articles containing MDA."

(7) If products containing MDA are exempted under subrules (2) to (6) of this rule, the employer shall maintain records of the initial monitoring results or objective data supporting that exemption and the basis for the employer's reliance on the data, as provided in the recordkeeping provision of 29 CFR 1926.60(o).

(8) These rules do not apply to general industry.  Exposure to MDA in general industry is covered by General Industry Safety and Health Standard Part 303. “Methylenedianiline (MDA) in General Industry.,” R 325.50051 to R 325.50076.

(9) The following federal Occupational Safety and Health Administration (OSHA) regulations are adopted by reference in these rules:

(a) 29 CFR 1926.60 “Methylenedianiline,” as amended April 11, 2018May 14, 2019.

(b) 29 CFR 1926.60, appendix A “Substance Data Sheet, for 4,4’-Methylenedianiline,” as amended June 20, 1996.

(c) 29 CFR 1926.60, appendix B “Substance Technical Guidelines, MDA,” as amended June 20, 1996.

(d) 29 CFR 1926.60, appendix C “Medical Surveillance Guidelines for MDA,” as amended June 20, 1996.

(e) 29 CFR 1926.60, appendix D “Sampling and Analytical Methods for MDA Monitoring and Measurement Procedures,” as amended June 20, 1996.

(10) A reference to 29 CFR 1910.133 means Construction Safety and Health Standard Part 6. “Personal Protective Equipment.,” R 408.40601 to R 408.40660.

(11)  A reference to 29 CFR 1910.38 means Construction Safety Standard Part 18. “Fire Protection and Prevention.,” R 408.41801 to R 408.41884.

(12)  A reference to 29 CFR 1910.1200 means Construction Safety Standard Part 42. “Hazard Communication.,” R 408.44201 to R 408.44204.

(13) A reference to 29 CFR 1910.141 means General Industry Safety and Health Standard Part 474. “Sanitation.,” R 325.47401 to R 325.47425

(14)  A reference to 29 CFR 1910.1020 and 1926.33 means General Industry and Construction Safety and Health Standard Part 470. “Employee Medical Records and Trade Secrets.,” R 325.3451 to R 325.3476.

(15) A reference to 29 CFR 1910.134 means Occupational Health Standard Part 451. “Respiratory Protection.,” R 325.60051 to R 325.60052.

(16) The adopted federal regulations have the same force and effect as a rule promulgated under the Michigan occupational safety and health act, 1974 PA 154, MCL 408.1001 to 408.1094.

(17) The OSHA regulations adopted in these rules are available from the United States Department of Labor, Occupational Safety and Health Administration website, www.osha.gov, at no charge, as of the time of adoption of these rules.

(18) The regulations adopted in these rules are available for inspection at the Department of Licensing and Regulatory AffairsLabor and Economic Opportunity, MIOSHA, Regulatory Services SectionStandards and FOIA Section, 530 West Allegan Street, P.O. Box 3064330645, Lansing, Michigan, 48909-814348909-8145.

(19) The regulations adopted in these rules may be obtained from the publisher or the Department of Licensing and Regulatory AffairsLabor and Economic Opportunity, MIOSHA, Regulatory ServicesStandards and FOIA Section, 530 West Allegan Street, P.O. Box 3064330645, Lansing, Michigan, 48909-814348909-8145, at the cost charged in this rule, plus $20.00 for shipping and handling.

(20) The following Michigan Occupational Safety and Health Administration (MIOSHA) standards are referenced in these rules.  Up to 5 copies of these standards may be obtained at no charge from the Michigan Department of Licensing and Regulatory AffairsLabor and Economic Opportunity, MIOSHA, Regulatory ServicesStandards and FOIA Section, 530 West Allegan Street, P.O. Box 3064330645, Lansing, Michigan, 48909-814348909-8145 or via the internet at the following website: www.michigan.gov/mioshastandards.  For quantities greater then than 5, the cost, as of the time of adoption of these rules, is 4 cents per page.

(a) Construction Safety and Health Standard Part 6. “Personal Protective Equipment,” R 408.40601 to R 408.40660.

(b) Construction Safety Standard Part 18. “Fire Protection and Prevention,” R 408.41801 to R 408.41884.

(c) Construction Safety Standard Part 42. “Hazard Communication,” R 408.44201 to R 408.44204.

(d) General Industry Safety and Health Standard Part 303. “Methylenedianiline (MDA) in General Industry,” R 325.50051 to R 325.50076.

(e) General Industry Safety and Health Standard Part 474. “Sanitation,” R 325.47401 to R 325.47425.

(f) General Industry and Construction Safety and Health Standard Part 470. “Employee Medical Records and Trade Secrets,” R 325.3451 to R 325.3476.

(g) Occupational HealthGeneral Industry and Construction Safety and Health Standard Part 451. “Respiratory Protection,” R 325.60051 to R 325.60052.